❄  SAMBA CRYO

Cryostat Measurement Application

ETH Zürich — Intermag Lab

Project Documentation

April 2026

Developer: Jakub Adventures

Collaborator: Tobias Goldenberg

AI Assistant: Claude (Anthropic)

# 1. Overview

Samba Cryo is a forked branch of SAMBA v3, purpose-built for cryogenic MOKE measurements using the AttoDRY cryostat system. It shares the core scan engine, plotting, data browser, and calibration modules with the standard SAMBA, but replaces the room-temperature hardware controls (Beckhoff magnet, relay) with cryostat-specific controls and adds temperature sweep capability.

Key differences from standard SAMBA:

- Single setup: Cryo only (no Green/IR pill buttons in the action bar)

- CryoHardwarePanel: Keithley 6221 current source (left) + AttoDRY cryostat controls (right)

- Scan modes: Spatial + Field / Temperature (no DC Hysteresis, no TR-MOKE)

- Temperature Sweep: sweeps AttoDRY Temperature using the FIELD scan engine

- Cryo Monitor: standalone rolling-history dialog for temperatures, pressures, and heater powers

- Cryo-blue branding (#0080FE accents) and separate QSettings key

- KeithleyMixin: shared Keithley control code in keithley_mixin.py

- ReadbackWorker: TANGO polling on background QThread instead of GUI thread

- No demagnetization: removed (not applicable to superconducting magnets)

# 2. File Structure

## 2.1 Cryo-Specific Files

These files differ from the standard SAMBA branch:

| File | Lines | Purpose |
| --- | --- | --- |
| samba_cryo.py | 953 | Entry point, CryoMainWindow, ReadbackWorker QThread, named tab constants |
| panels_cryo.py | 237 | CryoHardwarePanel: inherits KeithleyMixin + AttoDRY controls |
| cryo_monitor.py | 203 | CryoMonitorDialog: rolling 60s plots with incremental rendering |
| keithley_mixin.py | 199 | Shared Keithley 6221 UI builder and KeithleyMixin methods |
| panels.py | 2128 | ConfigListPanel simplified, TrajectoryPanel/ScanlistPanel accept hw_panel_class |
| config.py | 331 | Cryo setup defaults, iterative _sanitize for safe JSON serialization |
| hardware.py | 266 | TANGO proxy cache, SimProxy, safe_read/write (demagnetize removed) |
| scan.py | 1194 | ScanRunner, ScanWorker, ScanlistWorker (auto-demagnetize removed) |

## 2.2 Shared Files

These files are identical to the main SAMBA branch and must be present:

| File | Lines | Purpose |
| --- | --- | --- |
| plot_widgets.py | 322 | Live2DWidget and Live1DWidget with throttled matplotlib rendering |
| calibration.py | 636 | CalibrationPanel: autofocus, jog buttons, time scan plotting |
| data_browser.py | 725 | DataBrowserPanel: HDF5 file listing, viewing, and overlay |
| script_console.py | 263 | Embedded Python scripting console for ad-hoc automation |
| device_registry.py | 596 | Device Registry panel and JSON persistence |
| play_intro.py | 88 | Splash screen on startup |

# 3. Architecture

## 3.1 Import / Dependency Order

config ← hardware ← scan

config + hardware ← panels ← keithley_mixin ← panels_cryo + cryo_monitor

All of the above ← samba_cryo

## 3.2 How CryoMainWindow Differs from MainWindow

CryoMainWindow is a standalone class, not a subclass of MainWindow. Key differences:

- Only loads the "Cryo" setup — no setup pill buttons, just a "❄ CRYO" label

- Passes hw_panel_class=CryoHardwarePanel to TrajectoryPanel and ScanlistPanel constructors

- Uses ReadbackWorker (QThread) for all TANGO device polling

- Named tab constants (TAB_TRAJECTORY, TAB_SCANLIST, etc.) replace magic indices

- Extracts _wire_worker() helper for shared ScanWorker signal wiring

- Config save errors caught by _safe_save() and displayed in status bar

- Button styles defined once in CRYO_STYLE (no inline overrides)

- QSettings key "SambaCryo" for independent window geometry

## 3.3 Hardware Panel Injection

TrajectoryPanel and ScanlistPanel accept an optional hw_panel_class parameter (defaults to HardwarePanel). CryoMainWindow passes CryoHardwarePanel, so each panel constructs the correct hardware panel directly in __init__. This replaces the previous _replace_hw_panel() approach of recursively walking the layout tree at runtime to swap widgets.

## 3.4 KeithleyMixin (keithley_mixin.py)

The Keithley 6221 controls were previously duplicated (~80 lines) between HardwarePanel and CryoHardwarePanel. Extracted into keithley_mixin.py:

- build_keithley_group(owner): builds Current Source QGroupBox, attaches widgets to owner

- KeithleyMixin: provides _read_keithley, _write_range, _write_amplitude, _write_compliance, _write_frequency

- _make_spin(): shared spinbox factory with select-all-on-focus and enter-to-commit

- set_ok(), set_err(), set_sim(): status label helper functions

## 3.5 ReadbackWorker (QThread)

Previously, _poll_readbacks() ran all TANGO reads on the GUI thread. If any call blocked, the UI froze. Replaced with:

- ReadbackWorker(QThread): background thread emitting attodry_readback, fallback_field, ac_monitor, stage_positions signals

- 400ms QTimer (_sync_readback_state) pushes GUI state into the worker

- Signal handlers on CryoMainWindow update UI from the main thread

- Worker thread cleanly stopped in closeEvent() before saving config

## 3.6 ConfigListPanel Simplification

Previously maintained a hidden QTabWidget with tabs for Green, IR, Cryo. Replaced with a single QListWidget — no tabs, no SETUP_NAMES dependency. load_setups() takes the first setup from the dict and populates the list directly.

## 3.7 CryoMonitorDialog Rendering

Previously called ax.cla() on every 500ms poll for all 9 subplots. Now stores Line2D objects and uses line.set_data() + ax.relim() + ax.autoscale_view() for incremental updates. Significantly smoother performance.

# 4. Hardware & Device Map

The Cryo setup communicates with instruments via TANGO Controls:

| Device | TANGO Path | Type | Key Attrs |
| --- | --- | --- | --- |
| AttoDRY | hpp-N42/attoDRY/attoDRY | cryostat | MagneticField, Temperature, VtiTemp, MagnetTemp, ... |
| Keithley 6221 | hpp-N42/current/PyKeithley2 | current src | amplitude, frequency, compliance, range |
| ZI2 (lock-in) | hpp-N42/measure/ZI2 | lockin | x1, y1, x2, y2 |
| ZI1 | hpp-N42/measure/ZI1 | lockin | x1, y1, x2, y2 |
| Magnet | hpp-N42/beckhoff/magnet | magnet | field_polar_corr, current_polar |
| IR Stage | smaract2/control/IR-controller | stage | x, y, z |

## 4.1 AttoDRY Control Commands

- toggleMagneticFieldControl — enables/disables active field control on the superconducting magnet

- toggleFulltemperatureControl — enables/disables the temperature PID controller

- togglePersistentMode — switches between persistent and driven magnet modes

Note: The demagnetization routine has been removed. Alternating-decay demagnetization is not applicable to superconducting magnets.

# 5. Scan Modes

## 5.1 Spatial Scan

Identical to standard SAMBA. Moves one or two actuators (typically SmarAct stages X/Y) and reads sensors at each point. Supports 1D, 2D, and time-scan (no axes enabled).

## 5.2 Field Sweep

Sweeps a magnetic field and reads sensors at each point. The Field Sweep group has both Device and Attribute dropdowns. Multi-segment field list allows complex sweep patterns (e.g., -2→2, 2→-2, -2→0).

## 5.3 Temperature Sweep

Sweeps the AttoDRY Temperature attribute using the FIELD scan engine. Writes temperature setpoint, waits for configured settle time (60–300s), then reads sensors. Parameters: Device, Attribute, Start (K), Stop (K), N points, Settle time (s).

# 6. Cryo Monitor Dialog

Standalone QDialog with three columns of rolling matplotlib subplots (60s window, 500ms poll):

- Temperatures (K): Sample T, VTI T, Magnet T, Reservoir T

- Pressures (mbar): CryostatIn and CryostatOut

- Heater Powers (W): Sample, VTI, Reservoir

Rendering uses incremental line.set_data() updates. Dialog persists when closed (WA_DeleteOnClose is False).

# 7. CryoHardwarePanel

Replaces HardwarePanel via constructor injection. Inherits from KeithleyMixin and QGroupBox.

### Left: Current Source (Keithley 6221)

Built by build_keithley_group() from keithley_mixin.py. Range, Compliance, Amplitude (auto On/Off), Frequency spinboxes. Control methods inherited from KeithleyMixin.

### Right: AttoDRY Cryostat

Field setpoint (±9 T), Temperature setpoint (0–400 K), VTI/Magnet readbacks, toggle buttons (Mag Ctrl, Temp Ctrl, Persist), Monitor button. Exposes update_field_readback() and update_cryo_readbacks() with Optional[float] type annotations.

# 8. Configuration

## 8.1 Setup Defaults

The Cryo setup includes attodry_device: "hpp-N42/attoDRY/attoDRY". magnet_device and relay_device are empty.

## 8.2 Persistence

Configs saved as JSON in ~/.config/moke_scan/Cryo.json. save_setup() uses iterative _sanitize() to handle numpy/Qt types. Config save errors displayed in status bar via _safe_save().

## 8.3 Device Registry

Includes AttoDRY entry with all channels. Delete ~/.config/moke_scan/device_registry.json to load defaults.

# 9. Installation & Usage

## 9.1 Requirements

pip install pytango PyQt6 matplotlib h5py numpy

## 9.2 Running

export TANGO_HOST=192.168.1.1:10000

python samba_cryo.py

Without pytango, runs in simulation mode using SimProxy.

## 9.3 Files Required

All 14 Python files must be in the same directory: samba_cryo.py, panels.py, panels_cryo.py, keithley_mixin.py, cryo_monitor.py, config.py, device_registry.py, scan.py, hardware.py, plot_widgets.py, calibration.py, data_browser.py, script_console.py, play_intro.py.

# 10. Differences from Standard SAMBA

| Feature | Standard SAMBA | Samba Cryo |
| --- | --- | --- |
| Setup selector | Green / IR / Cryo pills | ❄ CRYO label only |
| Config list | Tabbed per setup | Single flat list |
| HardwarePanel | Keithley + Field & Relay | Keithley (KeithleyMixin) + AttoDRY |
| HW panel injection | Standard constructor | hw_panel_class parameter |
| Scan type pills | Spatial ¦ Field ¦ TR-MOKE | Spatial ¦ Field / Temperature |
| Demagnetization | Auto after field scans | Removed (superconducting) |
| Field readback | Beckhoff polling (GUI thread) | AttoDRY via ReadbackWorker QThread |
| Keithley code | Duplicated in both panels | Shared KeithleyMixin |
| Exception handling | Bare except: clauses | except Exception: with logging |
| Config save errors | Silently swallowed | Displayed in status bar |
| JSON serialization | Standard json.dump | Iterative _sanitize() |
| Tab indices | Magic numbers | Named constants |
| Branding | Catppuccin #89b4fa | Cryo blue #0080FE |
| Button styles | Duplicated inline | Single CRYO_STYLE source |

# 11. Refactoring Changelog

Summary of all structural improvements:

- Bare except → except Exception with proper logging

- Duplicated ScanWorker wiring → _wire_worker() helper

- Duplicated Keithley code → KeithleyMixin + build_keithley_group()

- ax.cla() every 500ms → line.set_data() incremental updates

- Inline stylesheet duplication → single CRYO_STYLE source

- Unused threading import removed from panels_cryo.py

- Added Optional[float] type annotations to readback methods

- Magic tab indices → named constants (TAB_TRAJECTORY, etc.)

- GUI-thread polling → ReadbackWorker(QThread)

- Silent config save → _safe_save() with status bar feedback + _sanitize()

- Fragile _replace_hw_panel → hw_panel_class constructor injection

- Demagnetization removed from hardware.py, scan.py, and panels.py

- ConfigListPanel: multi-setup QTabWidget → single QListWidget